Network topoplogy for a Bank

Name: khawlah alqas

Major: cyber security

Subject; advanced networks

Supervisor: DR/ Ameen

Introduction

This document provides the full design, configuration, and explanation of the Bank’s computer network. The network consists of a Headquarters (HQ) and two remote branches (Sector 1 and Sector 2 ) connected through WAN serial links. The network is segmented into VLANs to separate IT, HR, servers, and accounting traffic, ensuring scalability, security, and efficient resource usage.

The ISP connection provides simulated internet access, and NAT/PAT is used to translate private IP addresses for external communication. DHCP servers dynamically assign IPs to end devices across all VLANs.

- Network Topology Overview

- HQ (Headquarters)

  - Router-HQ (Cisco 2911)

  - Switch-HQ (Cisco 2960)

  - PCs (Admin, HR, IT)

  - Servers (Application Server)

  - Printer (IT)

  - WAN connections to Sector 1, Sector 2, and ISP

- Sector 1

  - Router-1(Cisco 2911)

  - Switch-1 (Cisco 2960)

  - PCs (IT, Accounting)

  - Printer (IT)

  - Labtop (Accounting)

  - WAN serial link to HQ

- Sector 2

  - Router-2 (Cisco 2911)

  - Switch-2 (Cisco 2960)

  - PCs (HR, IT, accounting)

  - Printer (HR)

  - Labtop (IT)

  - WAN serial link to HQ

- ISP Simulation

  - ISP Router (Cisco 2911)

  - ISP Switch (Cisco 2960)

  - Internet Server (Public address)

  - PC

  - printer

- IP Addressing Scheme

- compact table:

| Site / Network | Network (CIDR) | Mask | Gateway | Usable hosts |
| --- | --- | --- | --- | --- |
| HQ HR dept (VLAN25) | 192.168.110.0/29 | 255.255.255.248 | 192.168.110.1 | .7 |
| HQ IT (VLAN35) | 192.168.110.8/29 | 255.255.255.248 | 192.168.110.9 | .7 |
| HQ Servers (VLAN45) | 192.168.110.16/29 | 255.255.255.248 | 192.168.110.17 | .7 |
| HQ Accountonh (VLAN55) | 192.168.110.24/29 | 255.255.255.248 | 192.168.110.25 | .7 |
| Sector 1 HR dept | 192.168.111.0/29 | 255.255.255.248 | 192.168.111.1 | 7 |
| Sector 1 IT | 192.168.111.8/29 | 255.255.255.248 | 192.168.111.9 | 7 |
| Sector 1 accounting | 192.168.111.16/29 | 255.255.255.248 | 192.168.111.17 | 7 |
| Sector 2 IT | 192.168.112.0/29 | 255.255.255.248 | 192.168.112.1 | .7 |
| Sector 2 accounting | 192.168.112.8/29 | 255.255.255.248 | 192.168.112.9 | .7 |
| WAN HQ–ISP | 209.165.201.0/29 | 255.255.255.248 | 209.165.201.1 | .7 |
| WAN HQ–S1 | 10.7.7.0/30 | 255.255.255.252 | 259.165.251.1 | .4 |
| WAN HQ–S2 | 10.10.7.0/30 | 255.255.255.252 | 259.165.251.1 | .4 |
| ISP Public | 209.165.200.0 | 255.255.255.0 | 209.165.200.1 | .1–.254 |

Internet Simulation:

- ISP Network: 209.165.200.0/24 (TEST-NET-3)

- ISP Gateway: 209.165.200.1

- Internet Server: 209.165.200.11

- Routing Protocols

- Static Routing: Used for default routes pointing to the ISP.

- OSPF (Single Area 0): Used for dynamic routing between HQ, Branch A, and Branch B to ensure reachability across all subnets.

- DHCP Configuration

Each branch and HQ uses DHCP pools on their routers to dynamically assign IP addresses to end devices in VLANs. This reduces administrative overhead and ensures smooth client connectivity.

Example:

- VLAN 25 (HR) → DHCP range: 192.168.110.0– 192.168.110.7

- VLAN 35 (IT) → DHCP range: 192.168.110.8 – 192.168.110.15

- Access Control Lists (ACLs)

Two ACLs are configured for traffic control and security:

- Standard ACL (Layer 3):

  - Permits IP address list that was used NAT configuration.

- Extended ACL (Layer 3):

  - sDeny HR (VLAN 25) from using HTTP protocol AND HTTPS with servers

  - Permit all other traffic.

- NAT and PAT

- Static NAT: Used to map the internal Application Server (192.168.110.x) to a public IP on the ISP network.

- PAT (Overload): Used for all internal devices to share a single public IP when accessing the internet. This conserves address space and allows multiple users to connect simultaneously.

- Security Measures

- VLAN Segmentation: Ensures isolation of sensitive data (IT, HR, Servers).

- Port Security: Configured on all switch access ports to prevent MAC address spoofing.

  - Restrict to 1 MAC address per port (PC/Printer).

  - Violation mode set to shutdown.

- ACLs: Control access to critical resources and external servers.

- Cable Types

- Router ↔ Switch (HQ) → Copper Straight-Through (Gigabit to Gigabit)

- Router ↔ Router (WAN) → Serial DCE/DTE Cable

- Switch ↔ PCs/Servers/Printers → Copper Straight-Through

- Expected Behavior of the Network

- Inter-VLAN Communication: Achieved through Router-on-a-Stick configuration at HQ and branch routers.

- Branch Connectivity: OSPF ensures Sector 1 and Sector 2  can communicate with HQ and each other.

- Internet Access: All PCs can access the internet through HQ router NAT/PAT, except restricted traffic (VLAN 25).

- Server Hosting: HQ server is accessible internally and externally via static NAT.

- DHCP Dynamic Allocation: End devices in all VLANs automatically receive IPs.

- Access Restrictions: HR cannot access the external Internet Server or any other VLAN, while HR/Admin/Servers can.

Conclusion

The school’s network is designed with a balance of functionality, scalability, and security. VLANs ensure logical separation, DHCP eases address management, OSPF provides dynamic routing, ACLs enforce policies, and NAT/PAT manages internet access efficiently. This network is robust enough to handle current requirements while scalable for future growth.